# Cleopatra Documentation

# RTL Files Description

## spatz_dimc.sv

### Configuration

| Parameter | Allowed values | Description |
| --- | --- | --- |
| SECTION_WIDTH | 256, 512, or 1024 bits | Sets the section size. A 1024-bit kernel row and feature vector contain 4, 2, or 1 sections respectively. |

### Properties

| Storage | Size | Role |
| --- | --- | --- |
| Kernel memory | 32 × 1024 bits (32,768 bits) | Holds the weights matrix as 32 weight-vector rows. |
| Feature buffer | 1024 bits | Holds the input feature vector used in computation. |

### Internal Behavior

Memory mode: Kernel reads and feature-buffer loads use RCK; kernel writes use the independent WCK port. Kernel reads and writes may occur concurrently when COMPE=0.

Compute mode: The selected kernel row and input vector enter a four-stage pipeline. The computation is triggered when COMPE=1 and RCSN and RCSN0 to RCSN3 are all low.

### Pipeline

| Cycle | Stage | Operation |
| --- | --- | --- |
| N | Request | Computation request made. |
| N+1 | 0 — Input | Capture and assemble the kernel row, input vector, mode, mask, and ADDIN. |
| N+2 | 1 — Masking | Mask the selected operand bits. |
| N+3 | 2 — Computation | Perform vector multiplication. |
| N+4 | 3 — Output | Add ADDIN, register PSOUT and SOUT, and drive READYN low to indicate a valid result. |

The result becomes available at the output ports for sampling at N+4.

### Output Processing

PSOUT is the 32-bit computed result plus ADDIN. SOUT applies ReLU followed by clipping to 8 bits.

### Current Limitations

- Non-default SECTION_WIDTH values are not fully supported yet.

## spatz_dimc_dual.sv

### Structure

- 3 FIFO buffers

  - Input FIFO buffer

  - Weight FIFO buffer

  - Output FIFO buffer

- 2 DIMC macros sharing input and weight FIFO buffers

- 1 MUX that receives outputs of both DIMC macros and a select signal.

- The select signal decides which macro has its output passed to the output FIFO buffer

### Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| SECTION_WIDTH | 256, 512, or 1024 bits | Sets the section size. A 1024-bit kernel row and feature vector contain 4, 2, or 1 section respectively. |
| INP_FIFO_DEPTH | To be decided | Sets the depth of the input FIFO buffer |
| WGT_FIFO_DEPTH | To be decided | Sets the depth of the weight FIFO buffer |
| OUT_FIFO_DEPTH | To be decided | Sets the depth of the output FIFO buffer |

### Internal Behavior

- Signals sharing

  - Each macro has an independent set of control inputs.

  - clk and rst_n are the only shared control signals.

- Sel signal and MUX functionality

  - It does not gate commands.

  - D (kernel write data) is sent to both macros; each macro's independent WCSN/WEN controls whether it writes.

  - FD (feature data) is sent to both macros; each macro's independent FCSN controls whether it loads.

  - It chooses the macro whose output (PSOUT signal) is forwarded to the output FIFO.

- FIFOs auto management

  - FIFO pop and push signals are handled through combinational logic.

  - wgt_pop fires when either macro requests a write.

  - inp_pop fires when either macro requests a feature load.

  - out_push fires when the selected DIMC's READYN goes low (pipeline done).

- NOTE: simultaneous writes consume one shared FIFO word and presents it to both macros.

### Pipeline

- Similar to spatz_dimc module but the result becomes available at the output ports at N+5.

- This one clock cycle delay is needed to transfer the output signal from the DIMC macro output to the output FIFO.

### Current Limitations

- Non-default SECTION_WIDTH values are not fully supported yet.

- Parameters INP_FIFO_DEPTH WGT_FIFO_DEPTH, and OUT_FIFO_DEPTH are not decided yet.

# TB Files Description

## tb_spatz_dimc.sv

### Module Under Test

- Module is spatz_dimc, instantiated as i_dut with SECTION_WIDTH=256.

- All compute tests except Test 8 use sign_8b=00. Test 8 uses sign_8b = 01, 10, and 11.

  - sign_8b selects whether operands of kernel/feature are signed.

  - sign_8b =00 selects unsigned/unsigned

  - sign_8b =01 selects signed/unsigned

  - sign_8b =10 selects unsigned/signed

  - sign_8b =11 selects signed/signed

- All compute tests except Test 9 use MODE=11. Test 9 tests the other modes.

  - MODE selects precision of the feature and kernel values

  - MODE=00 selects 1-bit MAC operation.

  - MODE=01 selects 2-bit MAC operation.

  - MODE=10 selects 4-bit MAC operation.

  - MODE=11 selects 8-bit MAC operation.

### Stimulus Files Used

Base directory: stimuli/spatz_dimc_stims/

| File | Use | File organization |
| --- | --- | --- |
| kernel_weights.txt | Kernel data for Tests 2–8. | 128 lines: four consecutive sections for row 0, then row 1, through row 31. Sections are ordered 0–3. Each line is one 256-bit section containing 32 bytes; byte 0 is the least-significant byte. |
| feature_vector.txt | Input vector for Tests 5–8. | Four consecutive lines for sections 0–3 of one feature vector. Each line is one 256-bit section containing 32 bytes; byte 0 is the least-significant byte. |
| golden_clipped_8bit.txt | Expected SOUT values for Tests 5 and 6. | 32 lines ordered by kernel row 0–31. Each line is one 8-bit SOUT value written as two hexadecimal digits. |
| golden_psum_32bit.txt | Expected PSOUT values for Tests 5 and 6. | 32 lines ordered by kernel row 0–31. Each line is one 32-bit PSOUT value written as eight hexadecimal digits. |
| golden_with_masking_8bit.txt | Expected SOUT values for Test 7. | Six lines for kernel row 0, with no header or separators. Each line corresponds to one mask value, following the order in COMPUTE_MASK_VALS, and contains one clipped 8-bit SOUT value written as two hexadecimal digits. |
| golden_psum_with_masking_32bit.txt | Expected PSOUT values for Test 7. | Six lines for kernel row 0, with no header or separators. Each line corresponds to one mask value, following the order in COMPUTE_MASK_VALS, and contains one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. Negative values use two’s-complement representation. |
| golden_sign_XY_clipped_8bit.txt | Expected SOUT values for Test 8, sign_8b=XY. | 32 lines ordered by kernel row 0–31, with no header or separators. Each line is one clipped 8-bit SOUT value written as two hexadecimal digits. |
| golden_sign_XY_psum_32bit.txt | Expected PSOUT values for Test 8, sign_8b=XY. | 32 lines ordered by kernel row 0–31, with no header or separators. Each line is one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. Negative values use two’s-complement representation. |
| golden_mode_XY_clipped_8bit.txt | Expected SOUT values for Test 9, MODE=XY. | 32 lines ordered by kernel row 0–31, with no header or separators. Each line is one clipped 8-bit SOUT value written as two hexadecimal digits. |
| golden_mode_XY_psum_32bit.txt | Expected PSOUT values for Test 9, MODE=XY. | 32 lines ordered by kernel row 0–31, with no header or separators. Each line is one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. Negative values use two’s-complement representation. |

Files generator: All files are generated by stimuli/spatz_dimc_stim.py.

### Description of Tests

Each test resets the DUT and loads the kernel or feature data it requires, so tests can run independently.

#### Test 1 — Reset Verification

Purpose: Verify reset clears outputs, the feature buffer, and all kernel sections; READYN must remain high.

Stimulus/golden files: None logically required.

#### Test 2 — Kernel Write + Read-Back

Purpose: Write each kernel section separately, read it immediately through Q, and compare it with the stimulus.

Stimulus/golden files: kernel_weights.txt; the stimulus is also the expected value.

#### Test 3 — Optimized Kernel Write + Read-Back

Purpose: Write all kernel sections consecutively, then read and compare every section through Q.

Stimulus/golden files: kernel_weights.txt; the stimulus is also the expected value.

#### Test 4 — Concurrent Dual-Port Read + Write

Purpose: Read kernel row 0 section 0 through Q while writing row 0 section 1 through the independent write port, then verify both operations.

Stimulus/golden files: kernel_weights.txt; expected values are derived from the stimulus.

#### Test 5 — Full Matrix–Vector Multiplication (32 Rows)

Purpose: Load the kernel and feature vector, compute all 32 rows with compute_mask=0, and compare PSOUT and SOUT.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt, and golden_clipped_8bit.txt.

#### Test 6 — Pipelined Full Matrix–Vector Multiplication (32 Rows)

Purpose: Load required data, issue one computation per cycle, and compare all 32 consecutive PSOUT and SOUT results.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt, and golden_clipped_8bit.txt.

#### Test 7 — compute_mask Masking Sweep

Purpose: Load row 0 and the feature vector, compute six mask values, and compare PSOUT and SOUT.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_with_masking_32bit.txt, and golden_with_masking_8bit.txt.

#### Test 8 — Signed 8-Bit Multiplication

Purpose: Compute all 32 rows for sign_8b=01, 10, and 11, then compare PSOUT and SOUT with mode-specific golden values.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, and the six golden_sign_01, golden_sign_10, and golden_sign_11 PSOUT/SOUT files.

#### Test 9 — Multiplication with 1-bit, 2-bit and 4-bit precisions

Purpose: Compute MAC with modes of 1-bit, 2-bit, 4-bit precision for all kernel rows, and compare PSOUT and SOUT with precision-specific golden values.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, and the six golden_mode_01, golden_mode_10, and golden_mode_11 PSOUT/SOUT files.

### Current Limitations

• Tests cover only SECTION_WIDTH=256.

• All stimulus files are loaded unconditionally, even when their associated tests are disabled.

## tb_dimc_dual.sv

### Module Under Test

- Module is spatz_dim_dual, instantiated as i_dut with SECTION_WIDTH=256.

- All compute tests are on unsigned operations and 8-bit precision.

### Stimulus Files Used

Base directory: stimuli/spatz_dimc_stims/

- This tb reuses the same stimulus files used by tb_spatz_dimc.sv

Files generator: All files are generated by stimuli/spatz_dimc_stim.py.

### Description of Tests

#### Test 1 — Reset Verification

Purpose: Verifies that reset clears outputs, feature buffers, Q, and kernel SRAMs for both DIMCs, while READYN remains high.

Stimulus/golden files: None logically required.

#### Tests 2, 4 and Tests 3, 5 — Kernel Write + Read-Back

- Tests 2 and 4 are for DIMC 0 and tests 3 and 5 are for DIMC 1

Purpose: Writes all 32 kernel rows through weight FIFO and verifies by reading Q and comparing to golden model

Stimulus/golden files: kernel_weights.txt; the stimulus is also the expected value.

#### Test 6, 7, 8, 9 — Loading feature vector and performing dot product

- Tests 6 and 8 are for DIMC 0 and tests 7,9 are for DIMC 1

Purpose: Loads the feature vector into DIMC 0, computes 1 row dot product, and checks output of FIFO buffer against golden data.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt, golden_clipped_8bit.txt

#### Test 10 and 11 — Full MatVec multiplication

- Test 10 is for DIMC 0 and test 11 is for DIMC 1

Purpose: Computes all 32 rows on one DIMC macro without draining, then bulk-drains output FIFO and verifies result order/data against golden PSOUT values.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt, golden_clipped_8bit.txt

#### Test 12 and 13 — compute_mask Sweep on one DIMC

- Test 12 is for DIMC 0 and test 13 is for DIMC 1

Purpose: Sweeps six compute_mask values on row 0 of the DIMC macro and verifies PSOUT, clipped output, and FIFO order against masked golden files.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_with_masking_32bit.txt, golden_with_masking_8bit.txt

#### Test 14 — Overlapping Computes and verifying output FIFO output order

Purpose: Issues DIMC0 row 5 and then DIMC1 row 7 while the first compute is still in flight, and verifies both results arrive in output FIFO in issue order.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt.

#### Test 15 — Pipelined MatVec DIMC 0, POP All at End

Purpose: Reloads kernel/feature, issues one row compute per cycle for all 32 rows, then drains output FIFO at the end and compares all results to golden PSOUT.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt

#### Test 16 — Pipelined MatVec DIMC 0, POP Once Ready

Purpose: Reloads kernel/feature, issues one row compute per cycle, and pops/checks each output FIFO result as soon as it becomes ready against golden PSOUT.

Stimulus/golden files: kernel_weights.txt, feature_vector.txt, golden_psum_32bit.txt

### Current Limitations

- Tests cover only SECTION_WIDTH=256.

- All stimulus files are loaded unconditionally, even when their associated tests are disabled.

# Stimulus Files Description

## spatz_dimc_stim.py

### General Purpose

Generates reproducible input data and golden results for tb_spatz_dimc.sv using unsigned 8-bit kernel and feature values.

### Configurable Values

| Value | Default | Effect |
| --- | --- | --- |
| BIAS | 0 | Bias added to each expected multiplication result. |
| SECTION_WIDTH | 256 bits | Section size used to derive the number of sections and bytes per section. |
| COMPUTE_MASK_VALS | 0, 512, 768, 896, 960, 992 | Mask values used to generate Test 7 golden results. |
| --seed | 42 | Controls reproducible random kernel and feature values. |
| --outdir | stimuli | Selects the output directory. |
| SIGNED_8B_MODES | 01, 10, 11 | Selects the signed 8-bit modes for which Test 8 golden files are generated. |

### Generated Files

Base directory: stimuli/spatz_dimc_stims/

| File | Contents |
| --- | --- |
| kernel_weights.txt | A 32 × 128-element kernel matrix: 128 hexadecimal 256-bit sections. |
| feature_vector.txt | One 128-element input vector: four hexadecimal 256-bit sections. |
| golden_clipped_8bit.txt | 32 lines ordered by kernel row 0–31, with no header or separators. Each line contains one clipped 8-bit SOUT value written as two hexadecimal digits. |
| golden_psum_32bit.txt | 32 lines ordered by kernel row 0–31, with no header or separators. Each line contains one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. |
| golden_with_masking_8bit.txt | Six lines for kernel row 0, with no header or separators. Lines follow COMPUTE_MASK_VALS order; each contains one clipped 8-bit SOUT value written as two hexadecimal digits. |
| golden_psum_with_masking_32bit.txt | Six lines for kernel row 0, with no header or separators. Lines follow COMPUTE_MASK_VALS order; each contains one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. Negative values use two’s-complement representation. |
| golden_sign_XY_clipped_8bit.txt | 32 lines ordered by kernel row 0–31 for kernel-feature multiplication, with no header or separators. Each line contains one clipped 8-bit SOUT value written as two hexadecimal digits. XY refers to whether they are signed. |
| golden_sign_XY_psum_32bit.txt | 32 lines ordered by kernel row 0–31 for kernel-feature multiplication, with no header or separators. Each line contains one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. Negative values use two’s-complement representation. XY refers to whether they are signed. |
| golden_mode_XY_clipped_8bit.txt | 32 lines ordered by kernel row 0–31 for kernel-feature multiplication, with no header or separators. Each line contains one clipped 8-bit SOUT value written as two hexadecimal digits. XY refers to quantization precision used. |
| golden_mode_XY_psum_32bit.txt | 32 lines ordered by kernel row 0–31 for signed-kernel, unsigned-feature multiplication, with no header or separators. Each line contains one pre-clipping 32-bit PSOUT value written as eight hexadecimal digits. Negative values use two’s-complement representation. XY refers to quantization precision used. |